11/28/2016

Project 3

Sequence Recognizer 1001

Bibodi, Jay Nikhil

Sec 2: 5:30-6:45

Fall 2016

1. Consider problem 5.10

- Moore’s Sequence Recognizer for overlapping sequence 1001

- Detailed block diagram

- Truth table for OG

| State Name | Current State |  |  | Output |
| --- | --- | --- | --- | --- |
|  | q2 | q1 | q0 | Z |
| A | 0 | 0 | 0 | 0 |
| B | 0 | 0 | 1 | 0 |
| C | 0 | 1 | 0 | 0 |
| D | 0 | 1 | 1 | 0 |
| E | 1 | 0 | 0 | 1 |
|  | d | d | d | d |

- Truth table for NSG

| State Name | Current State |  |  | Input | Next State |  |  | Next State Name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | q2 | q1 | q0 | x | d2 | d1 | d0 |  |
| A | 0 | 0 | 0 | 0 | 0 | 0 | 0 | A |
| A | 0 | 0 | 0 | 1 | 0 | 0 | 1 | B |
| B | 0 | 0 | 1 | 0 | 0 | 1 | 0 | C |
| B | 0 | 0 | 1 | 1 | 0 | 0 | 1 | B |
| C | 0 | 1 | 0 | 0 | 0 | 1 | 1 | D |
| C | 0 | 1 | 0 | 1 | 0 | 0 | 1 | B |
| D | 0 | 1 | 1 | 0 | 0 | 0 | 0 | A |
| D | 0 | 1 | 1 | 1 | 1 | 0 | 0 | E |
| E | 1 | 0 | 0 | 0 | 0 | 1 | 0 | C |
| E | 1 | 0 | 0 | 1 | 0 | 0 | 1 | B |
| - | d | d | d | d | d | d | d | - |

d – don’t Cares in the truth table

- Logic Expression for output and next state variables.

- Final Circuit for FSM

- Behavioral Model for OG. (OG.v)

module OG

(

input q2,

output  z

);

assign z = q2;

endmodule

- Behavioral Model for NSG. (NSG.v)

module NSG

(

input [2:0] q,

input x,

output [2:0] d

);

assign d[2] = q[1] & q[0] & x;

assign d[1] = (q[2] & ~x) | (q[1] & ~q[0] & ~x) | (~q[1] & q[0] & ~x);

assign d[0] = (~q[1] & x) | (q[1] & ~q[0]);

endmodule

- Behavioral Model for FF. (FlipFlop.v)

module FlipFlop

(

input clock,reset,

input [2:0] nextState,

output reg [2:0] currentState

);

always@(posedge clock,posedge reset)

begin

if(reset == 1)

currentState <= 3'b000;

else

currentState <= nextState;

end

endmodule

- Combined Models to create FSM  (Project.v)

`include "OG.v";

`include "FlipFlop.v";

`include "NSG.v";

module Project

(

input clock,reset,x,

output out

);

wire [2:0] currentState, nextState;

NSG a(currentState,x,nextState);

FlipFlop b(clock,reset,nextState,currentState);

OG h(currentState[2],out);

endmodule

- TestBench (TestBench.v)

`include "Project.v";

module TestBench();

reg clock,reset,x;

wire z;

Project ProjectOne(clock,reset,x,z);

initial begin

$monitor("%4d:		z = %b",$time,z);

clock=0;

reset=1;      //Resets the FlipFlop

x=0;

#10 reset=0;      //end reset

end

always

begin

#5clock=~clock; //generates a clock signal with period 10

end

initial begin  // One input per clock cycle

#10 x = 0; $display ("%4d:	  x = %b",$time,x);

#10 x = 0; $display ("%4d:    x = %b",$time,x);

#10 x = 1; $display ("%4d:    x = %b",$time,x);

#10 x = 0; $display ("%4d:    x = %b",$time,x);

#10 x = 0; $display ("%4d:    x = %b",$time,x);

#10 x = 1; $display ("%4d:    x = %b",$time,x);

#10 x = 1; $display ("%4d:    x = %b",$time,x);

#10 x = 0; $display ("%4d:    x = %b",$time,x);

#10 x = 0; $display ("%4d:    x = %b",$time,x);

#10 x = 1; $display ("%4d:    x = %b",$time,x);

#10 x = 0; $display ("%4d:    x = %b",$time,x);

#10 x = 0; $display ("%4d:    x = %b",$time,x);

#10 x = 1; $display ("%4d:    x = %b",$time,x);

#10 $finish;

end

endmodule

Output:

2. Considered Problem 5.11

a.  Mealy’s Sequence Recognizer for overlapping sequence 1001

module MealyProject												(																input clock,reset,x,													output reg z													);

//assign binary encoded codes to the states A through D

parameter	A = 3'b000,														B = 3'b001,														C = 3'b010,														D = 3'b011;

reg[2:0] current_state, next_state;

//Section 1: Next State Generator (NSG)									always@(*)														begin																casex (current_state)  // ignore unknown and high impedence (Z) inputs							A: 	if(x == 1)															next_state = B;												else																next_state = A;											B: 	if(x == 1)															next_state = B;												else																next_state = C;											C: 	if(x == 1)															next_state = B;												else																next_state = D;											D: 	if(x == 1)															next_state = B;												else																next_state = A;											default: 																next_state = 3'bxxx;										endcase

end

//Section 2: Output Generator (OG)

always@(*) 														begin																if((x == 1) && (current_state == D) && (next_state == B )) 									z = 1'b1;													else																z = 1'b0;												end

//Section 3: FlipFlop

always@(posedge clock,posedge reset)										begin																if(reset ==1)															current_state <= A;												else 																current_state <= next_state;										end

endmodule

b. TestBench (MealyTestBench.v)

`include "MealyProject.v";

module MealyTestBench();												reg clock,reset,x;													wire z;

MealyProject projectTwo(clock,reset,x,z);

initial begin												$monitor("%4d:		z = %b",$time,z);									clock=0;												reset=1;      //Resets the FlipFlop										x=0; 															#10 reset=0;      //end reset											end

always														begin													#5clock=~clock; //generates a clock signal with period 10							end

initial begin  // One input per clock cycle									#10 x = 0; $display ("%4d:	  x = %b",$time,x);									#10 x = 0; $display ("%4d:    x = %b",$time,x);								#10 x = 1; $display ("%4d:    x = %b",$time,x);								#10 x = 0; $display ("%4d:    x = %b",$time,x);								#10 x = 0; $display ("%4d:    x = %b",$time,x);									#10 x = 1; $display ("%4d:    x = %b",$time,x);								#10 x = 1; $display ("%4d:    x = %b",$time,x);									#10 x = 0; $display ("%4d:    x = %b",$time,x);									#10 x = 0; $display ("%4d:    x = %b",$time,x);									#10 x = 1; $display ("%4d:    x = %b",$time,x);									#10 x = 0; $display ("%4d:    x = %b",$time,x);								#10 x = 0; $display ("%4d:    x = %b",$time,x);									#10 x = 1; $display ("%4d:    x = %b",$time,x);								#10 $finish;													end

endmodule

Output