10/19/2016

Project 2

Design an 8-bit Fast Adder

Bibodi, Jay Nikhil

Sec 2: 5:30-6:45

Fall 2016

Block Diagram for myXOR

Equation:

Block diagram of 2 bit CLA Adder with pgu, cgu and su units

Equations:

Block diagram for carry generate unit (CGU)

Equations:

Block Diagram for 8bitclaHybridAdder

Block diagram of 8 bit adder/subtractor

HDL Designs

myXOR.v

module myXOR(x,y,f);											                    input x,y;													   output f;

wire xor1, xor2, xor3, xor4;

not  xxor1(xor1,x);											                      not  yxor2(xor2,y);

nand yxor1(xor3,xor1,y);											      nand xxor2(xor4,xor2,x);										                   nand xor34(f,xor3,xor4);

endmodule

pg_su.v

`include "cgu.v"										                            module pg_su(a,b,ci,s,c);

input [1:0] a,b;												         input ci;												               output [1:0] s,c;

wire [1:0] pwire, gwire;

myXOR myXOR0(a[0],b[0],pwire[0]);									              myXOR myXOR1(a[1],b[1],pwire[1]);

and gwire0(gwire[0],a[0],b[0]);										       and gwire1(gwire[1],a[1],b[1]);

cgu cgu1(pwire,gwire,ci,c);

myXOR myXOR2(pwire[0],ci,s[0]);								                            myXOR myXOR3(pwire[1],c[0],s[1]);

endmodule

cgu.v

module cgu(p,g,ci,c);

input [1:0] p,g; 												     input ci;												                output [1:0] c;

wire nor1,nor2,nor3,nor4,nor5;

nor g0ci(nor4,g[0],ci);											                     nor g0p0(nor5,g[0],p[0]);										                     nor nor45(c[0],nor4,nor5);

nor g1g0ci(nor1,g[1],g[0],ci);											         nor g1g0p0(nor2,g[1],g[0],p[0]);										        nor g1p1(nor3,g[1],p[1]);

nor nor123(c[1],nor1,nor2,nor3);

endmodule

cla2bitAdder.v

`include "pg_su.v"											              module cla2bitAdder												             (

input [1:0] a,b,												      input ci,												          output [1:0] s,c

);

pg_su pg_su1(a,b,ci,s,c);

endmodule

cla8bitAdder.v

`include "cla2bitAdder.v"										              module cla8bitAdder											                           (

input [7:0] a,b,											               	input cin,											                	output [7:0] s, c											           );

cla2bitAdder unit1(a[1:0],b[1:0],cin,s[1:0],c[1:0]);							           cla2bitAdder unit2(a[3:2],b[3:2],c[1],s[3:2],c[3:2]);						                 cla2bitAdder unit3(a[5:4],b[5:4],c[3],s[5:4],c[5:4]);							    cla2bitAdder unit4(a[7:6],b[7:6],c[5],s[7:6],c[7:6]);

endmodule

invertor.v

module invertor												            (

input cin,										                           	input [7:0] b,											               	output reg [7:0] bout

);

always@(*)														begin																if (cin==1'b1)															bout = ~b;													else																bout = b;

end

endmodule

adderSubtractor.v

`include "cla8bitAdder.v"										              `include "invertor.v"											             `include "myXOR.v"

module adderSubtractor											             (

input [7:0] a,b,											               	input cin,													output [7:0] s,c,										              	output ovf												           );

wire [7:0] binvertor;

invertor invt1(cin,b[7:0],binvertor);									         cla8bitAdder add1(a,binvertor,cin,s,c);								              myXOR xyz(c[7],c[6],ovf);

endmodule

testBench.v

`include "adderSubtractor.v"										              module tester();

reg [7:0] a,b;													        reg cin;														       wire [7:0] s,c;													       wire ovf;

adderSubtractor adsub(a,b,cin,s,c,ovf);

initial begin										           a=8'hFF;b=8'h01;cin=1'b0;										                    #10 $display("a=%8b b=%8b cin=%b sum = %8b Overflow = %b C7 = %b C6 = %b ",a,b,cin,s,ovf,c[7],c[6]);

a=8'h7F;b=8'h01;cin=1'b0;									                                  #10 $display("a=%8b b=%8b cin=%b sum = %8b Overflow = %b C7 = %b C6 = %b ",a,b,cin,s,ovf,c[7],c[6]);

a=8'h01;b=8'hFF;cin=1'b0;											        #10 $display("a=%8b b=%8b cin=%b sum = %8b Overflow = %b C7 = %b C6 = %b ",a,b,cin,s,ovf,c[7],c[6]);

a=8'h55;b=8'hAA;cin=1'b0;											        #10 $display("a=%8b b=%8b cin=%b sum = %8b Overflow = %b C7 = %b C6 = %b ",a,b,cin,s,ovf,c[7],c[6]);

a=8'h80;b=8'h01;cin=1'b1;											       #10 $display("a=%8b b=%8b cin=%b sum = %8b Overflow = %b C7 = %b C6 = %b ",a,b,cin,s,ovf,c[7],c[6]);

a=8'h6C;b=8'hCA;cin=1'b1;										                     #10 $display("a=%8b b=%8b cin=%b sum = %8b Overflow = %b C7 = %b C6 = %b ",a,b,cin,s,ovf,c[7],c[6]);

a=8'hDD;b=8'h09;cin=1'b1;										                    #10 $display("a=%8b b=%8b cin=%b sum = %8b Overflow = %b C7 = %b C6 = %b ",a,b,cin,s,ovf,c[7],c[6]);

a=8'hEF;b=8'h11;cin=1'b0;											       #10 $display("a=%8b b=%8b cin=%b sum = %8b Overflow = %b C7 = %b C6 = %b ",a,b,cin,s,ovf,c[7],c[6]);

end													         endmodule

Simulation for adder/subtractor